SIMATS ENGINEERING

Department of Computer Science and Engineering

# ASSESSMENT TOOL 1 – INDUSTRY PROBLEM SOLVING TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO4 – Precision (BL4) | Assessment: | Industry Problem solving task |
| Weightage: | 30% | Total Marks: | 100 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | R. Hardhik Shankar | Reg. No.: | 192312417 |
| Section / Batch: | 2023 | Date: | 17/08/2026 |

Instructions to Students - Identify the relevant concepts of cache hierarchy, SRAM, DRAM, MRAM, NAND Flash, RRAM, memory latency, bandwidth, and virtual memory paging. - Analyze the memory-access requirements of smartphones, cloud servers, autonomous vehicles, edge AI devices, and gaming consoles, considering latency, bandwidth, capacity, and power consumption. - Apply suitable memory-hierarchy techniques such as cache optimization, appropriate memory technology selection, data locality, paging optimization, and hierarchical memory organization. - Compute performance measures such as memory access time, latency, bandwidth, throughput, speedup, hit/miss rates, and resource or power utilization where applicable. - Interpret the results by evaluating performance improvements, memory bottlenecks, technology trade-offs, and the suitability of the proposed memory hierarchy for each application.

QUESTIONS

- A mobile device manufacturer observes noticeable lag when users switch rapidly between multiple applications. Analyze how L1, L2, and L3 caches and DRAM participate in storing and retrieving frequently accessed application data. Based on latency, capacity, hit rate, and bandwidth requirements, propose a suitable hierarchical memory organization that can reduce applicationswitching delays and improve overall system throughput.

- A cloud service provider experiences increased latency when processing database queries from a large number of concurrent users. Analyze how cache hierarchy and virtual memory paging influence memory access performance, and compare their roles in reducing processor-memory latency. Recommend suitable memory technologies and memory-management improvements that can minimize page faults, reduce access delays, and improve database throughput.

- An autonomous vehicle continuously receives large volumes of sensor data from cameras, LiDAR, radar, and other real-time sensing systems. Evaluate the characteristics of SRAM, DRAM, and MRAM in terms of access latency, bandwidth, capacity, volatility, and power consumption. Based on the real-time processing requirements, design an appropriate memory hierarchy that can provide high data-transfer bandwidth while minimizing access latency and supporting reliable sensor-data processing.

- An edge-based AI device must load machine-learning models quickly while operating under strict power and storage constraints. Analyze the characteristics of NAND Flash, DRAM, and RRAM with respect to access speed, storage capacity, persistence, power consumption, and suitability for AI workloads. Recommend a hierarchical memory organization that enables fast model loading, efficient intermediate-data processing, reduced energy consumption, and improved overall inference performance.

- A high-performance gaming console experiences frame drops and noticeable delays when loading large game environments containing high-resolution textures, audio, and other assets. Analyze the interaction between L3 cache and DRAM during the retrieval and processing of frequently accessed game data, considering their latency, capacity, and bandwidth. Propose suitable memoryhierarchy enhancements that can reduce data-access bottlenecks, increase memory throughput, and provide smoother frame rendering during large scene transitions.

## Code of Conduct

I R. Hardhik Shankar (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the Student: _________________________

RUBRICS FOR CALCULATION

| Criteria | Weightage(%) | Level 4 – Exemplary (5 Marks) | Level 3 – Proficient (4 Marks) | Level 2 – Developing (2–3 Marks) | Level 1 – Beginning (0–1 Mark) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Industry Problem | 20% | Comprehensive understanding of real-world problem and constraints | Good understanding with minor gaps in requirements | Basic understanding; some requirements unclear | Poor understanding; misinterprets problem context |
| Application of Engineering Concepts | 25% | Applies advanced and relevant concepts effectively to solve the problem | Applies appropriate concepts with minor errors | Limited application of concepts | Incorrect or no application of concepts |
| Solution Design | 25% | Innovative, practical, and feasible solution aligned with industry standards | Logical and feasible solution with minor gaps | Basic solution with limited feasibility | Unclear or impractical solution |
| Use of Tools | 15% | Effective use of modern tools, technologies, and real data | Appropriate use of tools with correct implementation | Limited or inconsistent use of tools | Minimal or incorrect use of tools |
| Analysis | 10% | Thorough analysis with validated results and performance evaluation | Good analysis with reasonable validation | Basic analysis with limited validation | Poor or no analysis |
| Professionalism | 5% | Highly professional, well-documented, and clearly presented work | Good documentation and presentation | Basic documentation with some gaps | Poor documentation and presentation |

Marks Evaluation:

| Q. No | Understanding of Industry Problem (20) |  | Application of Engineering Concepts (25) |  | Solution Design (25) | Use of Tools (15) | Analysis (10) | Professiona lism(5) | Total (out of 100) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 2 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 3 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 4 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 5 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
|  | / 25 |  | / 30 |  | / 20 | / 15 | / 10 | /5 | / 100 |

Course Coordinator  	 	              	 	 	Faculty Incharge

- Mobile Device: Reducing Application-Switching Delay

When a user rapidly switches between applications, the processor repeatedly accesses application instructions and data. The L1, L2, L3 caches and DRAM work together as a hierarchy.

- L1 Cache: Smallest and fastest cache. It stores the most frequently accessed instructions and data. It provides very low latency and should have a high hit rate.

- L2 Cache: Larger than L1 but slightly slower. It stores data that does not fit in L1 and reduces accesses to slower memory.

- L3 Cache: Much larger and shared among processor cores. It stores frequently shared application data and reduces DRAM accesses.

- DRAM: Provides large capacity but has significantly higher latency than cache. It stores the active applications and their working data.

Proposed hierarchy

CPU → L1 → L2 → L3 → DRAM → Storage

The system should use relatively large L2/L3 caches and efficient cache replacement policies such as LRU or adaptive replacement. Frequently used application pages should remain in DRAM to avoid storage access.

A high cache hit rate reduces the average memory access time:

AMAT = Hit Time + (Miss Rate × Miss Penalty)

Therefore, improving L1/L2/L3 hit rates reduces application-switching delays. DRAM with sufficient bandwidth should support multiple applications simultaneously.

Result

This hierarchy provides:

- Low latency through L1/L2

- Higher shared capacity through L3

- Large working capacity through DRAM

- Reduced processor stalls

- Faster application switching

- Improved overall throughput

This directly addresses the assignment's requirement to consider cache capacity, hit rate, latency, and bandwidth.

2. Cloud Server: Database Query Latency

Cloud databases experience large numbers of simultaneous memory accesses. Both cache hierarchy and virtual memory paging affect performance, but they solve different problems.

Cache hierarchy

The cache hierarchy keeps frequently accessed database information close to the CPU.

L1 → L2 → L3 → DRAM

For example, frequently accessed indexes, query instructions, and metadata can remain in cache. A cache hit is much faster than accessing DRAM.

Virtual memory paging

Virtual memory allows applications to use more logical memory than the available physical RAM. Memory is divided into fixed-size pages.

When a required page is not present in physical memory, a page fault occurs. The operating system must retrieve it from secondary storage, causing a large delay.

Therefore:

Cache miss → relatively small penalty

Page fault → very large penalty

Recommended improvements

- Increase available DRAM capacity.

- Use high-bandwidth DDR memory.

- Increase effective L3 cache capacity.

- Use database buffer pools to keep frequently accessed records in RAM.

- Use LRU/working-set based page replacement.

- Reduce unnecessary swapping.

- Use huge pages where appropriate to reduce TLB overhead.

- Keep frequently accessed database indexes in memory.

- Use fast NVMe storage as backing storage.

Recommended hierarchy

CPU → L1 → L2 → L3 → DRAM → NVMe SSD

This minimizes expensive page faults while cache hierarchy minimizes normal processor-memory latency. Consequently, database throughput improves under high concurrent workloads. The assessment specifically asks for minimizing page faults and improving database throughput.

3. Autonomous Vehicle: Sensor-Data Processing

An autonomous vehicle receives continuous data from cameras, LiDAR, radar and other sensors, making both latency and bandwidth critical. The question specifically requires comparison of SRAM, DRAM and MRAM.

| Memory | Latency | Capacity | Volatile? | Power | Best Use |
| --- | --- | --- | --- | --- | --- |
| SRAM | Very low | Low | Yes | Relatively high per bit | CPU/cache, real-time buffers |
| DRAM | Low | High | Yes | Moderate | Large sensor-data storage |
| MRAM | Low | Medium | No | Low standby power | Persistent/critical data |

SRAM

SRAM is extremely fast and is suitable for:

- Processor caches

- Sensor processing buffers

- Frequently accessed real-time data

However, its high cost per bit makes large SRAM capacities impractical.

DRAM

DRAM offers much higher capacity and good bandwidth. It is suitable for:

- Camera frames

- LiDAR point clouds

- Radar data

- Intermediate AI-processing data

Its disadvantage is higher latency compared with SRAM and its volatile nature.

MRAM

MRAM is non-volatile, meaning it retains information without continuous power. It offers fast access and low standby power, making it useful for:

- Critical configuration data

- Persistent sensor information

- Safety-related information

- Fast recovery after power interruptions

Proposed hierarchy

CPU/AI Accelerator → SRAM → DRAM → MRAM → Storage

SRAM should hold the most time-critical working data. DRAM should store large sensor streams and intermediate processing results. MRAM can store persistent or safety-critical information.

Result

This architecture provides high bandwidth through DRAM, very low latency through SRAM, and persistence through MRAM, making it appropriate for real-time autonomous-vehicle processing.

4. Edge AI Device: Fast Model Loading

An edge AI device has strict limitations on power, storage capacity and processing speed. The assignment requires comparison of NAND Flash, DRAM and RRAM for AI workloads.

| Memory | Speed | Capacity | Persistence | Power | Suitable Use |
| --- | --- | --- | --- | --- | --- |
| DRAM | Very high | High | Volatile | Moderate | AI model execution |
| NAND Flash | Lower | Very high | Non-volatile | Low standby | Model/storage |
| RRAM | High | Medium | Non-volatile | Low | AI/near-memory processing |

NAND Flash

NAND Flash provides:

- High storage density

- Non-volatile storage

- Low standby power

- Low cost per bit

It is suitable for permanently storing large machine-learning models.

However, it is slower than DRAM for active computation.

DRAM

DRAM should hold:

- Loaded AI models

- Intermediate tensors

- Activation data

- Runtime buffers

It provides much higher bandwidth than NAND Flash.

RRAM

RRAM is a promising non-volatile memory technology that can provide high density and low-power operation. It can potentially support near-memory or in-memory AI processing, reducing data movement between processor and memory.

Proposed hierarchy

AI Accelerator → DRAM → RRAM → NAND Flash

A practical implementation can use:

NAND Flash: permanent model storage
↓
DRAM: active model and intermediate data
↓
RRAM: frequently reused parameters / AI acceleration

Frequently used models can remain cached in DRAM, avoiding repeated NAND Flash reads.

Result

The proposed hierarchy reduces model-loading time, minimizes unnecessary data movement and lowers energy consumption while improving AI inference performance.

5. Gaming Console: Reducing Frame Drops

Large game environments contain high-resolution textures, audio, geometry and other assets. During scene transitions, the CPU/GPU may need to retrieve large amounts of data from DRAM. The question specifically focuses on the interaction between L3 cache and DRAM.

Role of L3 Cache

L3 cache is much faster than DRAM and stores frequently reused data such as:

- Game-engine instructions

- Frequently accessed game-state information

- Physics data

- Frequently reused textures or metadata

When required data is found in L3:

CPU → L3 → Data

This avoids a slower DRAM access.

Role of DRAM

DRAM provides much greater capacity and stores:

- Large textures

- Audio assets

- Geometry

- Game-world data

- Runtime buffers

However, DRAM has higher latency than L3 cache.

Problem

If the working set is much larger than the L3 cache, the L3 miss rate increases, causing frequent DRAM accesses. During large scene transitions, this can create a memory-bandwidth bottleneck and cause frame-time spikes.

Proposed improvements

- Increase effective L3 cache capacity.

- Use efficient cache replacement policies.

- Improve spatial and temporal locality.

- Prefetch frequently required game assets.

- Use high-bandwidth DRAM.

- Optimize texture and asset layout.

- Stream game assets before they are required.

- Reduce unnecessary DRAM transfers.

Proposed hierarchy

CPU/GPU → L1/L2 → Large L3 → High-Bandwidth DRAM → SSD

Frequently accessed data remains in cache, while large assets remain in DRAM and are streamed from SSD when required.

Result

A larger and better-optimized L3 cache reduces DRAM accesses, while high-bandwidth DRAM handles large game assets. Together, they reduce memory bottlenecks, improve throughput and provide smoother frame rendering during large scene transitions.

Quick Comparison of the Five Solutions

Application; Main Problem; Best Memory Strategy. Smartphone; App-switching latency; Large/effective L2-L3 + sufficient DRAM. Cloud Server; Database latency/page faults; Large DRAM + cache optimization + paging optimization. Autonomous Vehicle; Huge real-time sensor bandwidth; SRAM + high-bandwidth DRAM + MRAM. Edge AI; Fast model loading + low power; NAND Flash + DRAM + RRAM. Gaming Console; Frame drops/data bottleneck; Large L3 + high-bandwidth DRAM + prefetching.